Syntax is:   device = prspf.sys [/P=1] /T1=3H

P=Port

1=LPT1:

2=LPT2:

T1=Type of 1st Drive.  T2=Type of 2nd drive.

3L=3.5" Low Density Drive      (720Kb)               /N:9/T:80

3H=3.5" High Density Drive     (720Kb/1.44Mb)        /N:18/T:80

3E=3.5" Extra High Density     (720Kb/1.44Mb/2.88Mb) /N:36/T:80

5L=5.25" Low Density Drive     (360Kb)        /N:9/T:40

5H=5.25" High Density Drive    (360Kb/1.2Mb)  /N:15/T:80

NOTES:

DR DOS 6.0

FORMATTING DISKETTES

In some situations when attempting to format diskettes an error

message may indicate that the diskette is formatted using another

system and/or the diskette is unusable.  It may be necessary

to use the following syntax: (assuming the drive is drive "D:")

720Kb 3.5"  diskettes - FORMAT D:/N:9/T:80

1.44mB 3.5" diskettes - FORMAT D:/N:18/T:80

360Kb 5.25" diskettes - FORMAT D:/N:9/T:40

1.2Mb 5.25" diskettes - FORMAT D:/N:15/T:80

To use this software on Hewlett-Packard's OmniBook 300, it must

first be transferred from another computer via Laplink into the

root directory of the boot drive. Then use the syntax above

to install into the config.sys file, or run the install program,

which will make the insertion for you.